THE NMC ETA MODEL HORIZONTAL OUTPUT GRID FOR GCIP/GIST

The chosen GCIP/GIST horizontal output grid from the

mesoscale NMC Eta Model is NMC Grid No. 212, known as

AWIPS Grid 212.  This grid is defined on a Lambert

Conformal projection with a nominal horizontal grid

resolution of 40-km and grid dimensions of 185 x 129.

The latitude/longitude locations of the corner points

of this grid are given below:

( I , J )           Latitude / Longitude

( 1 , 1 )           12.190 N / 133.459 W

( 1 ,129)           54.536 N / 152.856 W

(185,129)           57.290 N /  49.385 W

(185, 1 )           14.335 N /  65.091 W

The above domain covers most of N. America and the nearby

oceans, including the 48 contiguous U.S. states, the southern

half of Canada, and most of Mexico.

The latitude and longitude values for the entire 185 x 129

grid have been provided in two companion files; one file

written as a packed binary file and one file written as

a formatted ascii file.  These files are decribed below:

File lalog212.fmt :

Written as ascii formatted records (total records = 23865,

i.e. 185 x 129).  Each record contains the following

information:  I index value, J index value, latitude(I,J) (+N),

longitude(I,J) (deg E), written with this FORMAT statement:

(2I4,1X,F12.5,1X,F12.5)

File lalog212.grb :

Written as an unblocked packed binary file using the

WMO international "GRIB" packing convention.  Two arrays

written:

1) the latitudes  of the 185 x 129 grid (+N) and

2) the longitudes of the 185 x 129 grid (deg E)

For documentation and unpacking software for the GRIB

packing convention, see Sec. IV.b of the NMC On-line

GCIP documentation file called "gcip.doc" in directory

/pub/mso on the anonymous NMC Internet Server 140.90.50.22

(use ID: anonymous, Password: your Internet e-mail address)

A user can check his or her GRIB unpacking software by

unpacking file lalog212.grb and comparing the values with

the values is the preceding ascii file lalog212.fmt

******      IMPORTANT NOTE     ******

The NMC on-line file "gcip.doc" provides a comprehensive

description of all the NMC Eta Model output fields and data

formally provided for GCIP.  See above paragraph for

Internet access instructions.